Hunter Freeze-Clik® Sensor Written Specifications

Part 1 – General

1.1  The Freeze-Clik Sensor stops scheduled irrigation if it detects that ambient air temperature has fallen below 37°F (3°C) by interrupting the power from the irrigation controller to the valves via a sensor terminal with wired communication.

Part 2 – Parts and Material

- All Freeze-Clik Sensor elements shall be housed in UV- and corrosion-resistant plastic casing; the sensing technology shall be encased in epoxy.

Part 3 – Sensor Hardware

- Components

  - The Freeze-Clik Sensor has a built-in temperature sensor. It shall be capable of interrupting the power from the irrigation controller to the valves when ambient air temperature falls below 37°F (3°C).

  - The factory tolerance for the setpoint on the Freeze-Clik Sensor shall be ± 3.6°F (2°C).

- Warranty

  - The Freeze-Clik Sensor shall be installed in accordance with the manufacturer’s published instructions and shall carry a conditional five-year exchange warranty. The Freeze-Clik Sensor shall be as manufactured for Hunter Industries Incorporated, San Marcos, California.

Part 4 – Operating Specifications

- Hardware

  - The wired Freeze-Clik Sensor shall be mounted within 200' (60 m) of the irrigation controller. Wired models shall include 25' of 20 AWG (7.6 m of 0.8 mm2) sheathed, two-conductor, UL-approved wire.

  - If an extension to the wire is needed, the wire shall be no lighter than 18 AWG (1 mm2) sheathed, two-conductor, UL-approved wire.

  - The wired Freeze-Clik Sensor shall be mounted in a location free of direct sunlight exposure and where free outdoor air circulation is possible for the most accurate temperature sensing.

- General

  - The sensor shall be fully functional with 24 VAC input or less, with a current draw of 6 A, and shall not require a dedicated high-voltage transformer or connection.

© 2021 Hunter Industries™. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.