- Hunter Product Written Specification Template (MP)

Part 1 – General

  - General Description of Product and Function

The MP Rotator is a high-efficiency, multi-stream and multi-trajectory rotary nozzle designed with a viscous drive for rotation. The multiple streams of water allow for a slow application of water without misting in the pattern at a high distribution uniformity. The standard MP Rotator family, designated by the black canister, has a matched precipitation rate of approximately 0.4 in/hr across any arc and radius to better match soil intake rates and prevent runoff. The MP800 family, designated by the gray canister, has a matched precipitation rate of approximately 0.8 in/hr. MP Rotators have female threads for installation on male threaded pop-up sprinklers, and select models have the male-threaded option for installation on female threaded pop-up sprinklers. Each MP Rotator shall have a filter screen to prevent internal system debris from entering the nozzle and the patented double-pop feature to prevent external debris from falling into the nozzle. Each MP Rotator shall be color-coded for easy field identification.

Part 2 – Parts and Material

- The MP Rotator shall be available in the following options:

Standard MP Rotator Series (approximately 0.4 in/hr precipitation rate)

MP-1000-90, MP-1000-210, MP-1000-360 for an 8-15 ft radius when operating at 30-55 psi.

MP-2000-90, MP-2000-210, MP-2000-360 for a 13-21 ft radius when operating at 25-55 psi.

MP-23000-90, MP-3000-210, MP-3000-360 for a 22-30 ft radius when operating at 25-55 psi.

MP-3500-90 for a 31-35 ft radius when operating at 25-55 psi.

MP-CORNER for an 8-15 ft radius when operating at 25-55 psi.

MP Rotator Strip Series (precipitation rate dependent on layout)

MP-LCS-515, MP-RCS-515, MP-SS-530 for 5 ft wide strip models.

MP800 Series (approximately 0.8 in/hr precipitation rate)

MP-800SR-90, MP-800SR-360 for 6-12 ft radius when operating at 30-55 psi.

MP-815-90, MP-815-210, MP-815-360 for 8-15ft radius when operating at 30-55 psi.

- Plastic Material Description

- The adjustable orifice shall be manufactured from polyurethane and acetal plastic materials for durability and adjustability.

- The acetal material shall have UV stabilizers for outdoor applications.

- Metal Component Materials

- The radius adjustment screw, arc ring, spring, and internal collar shall be made of stainless steel.

- The stator that drives the speed of rotation inside the silicone chamber shall be made of brass.

- Filter Screen Description

- Each MP Rotator shall come with a detachable filter screen.

- The filter screens shall be made of polypropylene.

- Screen mesh size shall be dependent on the MP Rotator model.

  - 60 mesh: MP800SR90

  - 40 mesh: MP1000, MP2000, MP Corner, MP Strips, MP800SR360, MP815

  - 20 mesh: MP3000, MP3500

- Color Description

  - Each MP Rotator model shall have its own designated color scheme.

  - Standard MP Rotators have a black canister and black top retainer.

    - MP-1000-90 (maroon), MP-1000-210 (light blue), MP-1000-360 (olive)

    - MP-2000-90 (black), MP-2000-210 (green), MP-2000-360 (red)

    - MP-3000-90 (blue), MP-3000-210 (yellow), MP-3000-360 (gray)

    - MP-3500-90 (tan)

    - MP-Corner (turquoise)

    - MP-LCS-515 (ivory), MP-RCS-515 (copper), MP-SS-530 (brown)

  - The MP800 family has a gray canister and gray top retainer.

    - MP-800SR-90 (orange), MP-800SR-360 (lime green)

    - MP-815-90 (maroon), MP-815-210 (light blue), MP-815-360 (olive)

- Nozzle Threads

- Models MP1000, MP2000, MP3000, MP3500, MP Corner, MP Strips, MP800SR, and MP815 shall be able to be installed in pop-up bodies having a 5/8-27 UNS male threaded stem at all common pop-up heights.

- Models MP1000HT, MP2000HT, MP3000HT, MP Corner HT, MP Strip HT shall be able to be installed in pop-up bodies having a 5/8-28 UNS female threaded stem at all common pop-up heights.

- Viscous Drive

- The viscous fluid used to maintain the speed of rotation of the MP Rotator shall be made of a silicone material.

- The silicone chamber shall be sealed with EPDM rubber seals.

- The brass stator inside the silicone chamber shall control the speed of rotation.

  - Warranty

- 3 year warranty period

Part 3 – Function and Operation

- Operating Pressure

- The MP Rotator shall operate between 25-55 psi (1.7-3.8 bar; 170-380 kPa).

- The recommended operating pressure is 40 psi (2.8 bar; 280 kPa).

- Flow Rates

- The flow rates shall depend on the MP Rotator model.

- As the arc and radius are adjusted, the flow rate shall change to maintain matched precipitation.

  - Radius Description

- The radius of throw shall depend on the MP Rotator model.

- At the recommended 40 psi operating pressure, full or part circle sprinklers shall be capable of up to 25% radius reduction using a stainless steel radius adjustment screw.

- The radius reduction screw shall have a slip clutch mechanism to prevent internal damage if turned past the minimum or maximum radius settings

- The radius reduction screw shall reduce the pressure and flow upstream of the adjustable orifice thereby maintaining stream integrity.

- Arc Adjustment

- The part circle sprinkler shall have an infinitely adjustable arc from 45° to 105°, 90° to 210° or between 210° to 270° using the stainless steel arc ring depending on the model selected.

- The full circle sprinkler shall irrigate a full 360°.

- The 45° to 105° model shall not require coverage from adjacent sprinklers closer than 3’ from the head.

- Arc adjustment shall be effective only while the sprinkler is popped up and shall be ineffective when the sprinkler is popped down.

- When turned past the minimum or maximum arc limits the adjustment mechanism shall have a ratcheting action to prevent internal damage.

  - Application Rate

    - Models MP1000, MP2000, MP3000, MP3500, MP Corner, MP Strips shall produce and maintain a matched precipitation rate no greater than 0.6” per hour throughout the arc adjustment range and radius adjustment range, (up to 25% of radius reduction), when spaced at 50% of wetted diameter.

    - Models MP800SR and MP815 shall produce and maintain a matched precipitation rate no greater than 1.0” per hour throughout the arc adjustment range and radius adjustment range, (up to 25% of radius reduction), when spaced at 50% of wetted diameter.

  - Double-Pop

    - When installed in a pop-up sprinkler body, the MP Rotator rotor shall pop-up after the body stem is fully extended. Upon decreasing pressure, the MP Rotator rotor shall retract before the retraction of the sprinkler body stem.

    - The MP Rotator itself shall pop-up at approximately 15 psi (1.0 bar; 100 kPa).