- Hunter Product Written Specification Template (Pro-Spray)

Part 1 – General

  - General Description of Product and Function

The Hunter Pro-Spray is a sprinkler pop-up designed for residential and commercial applications in both pressure-regulated and non-pressure-regulated models. Standard Pro-Spray models shall be available with a 2, 3, 4, 6, and 12 inch (5, 7, 10, 15, and 30 cm) pop-up stroke as well as an aboveground shrub head. Pressure-regulated PRS30 and PRS40 models shall be available in a shrub model and a 4, 6, and 12 inch (10, 15, and 30 cm) pop-up stroke. Standard and PRS30 models in 6 and 12 inch (15 and 30 cm) shall have a side inlet option. Pop-up models shall have the option of a factory or field-installed drain check valve and a reclaimed water purple body cap. Pressure-regulated models shall have the option of the drain check valve and FloGuardTM technology to reduce the flow through the riser in the event of a missing nozzle. Factory-installed features shall be designated on the top of the Pro-Spray cap for easy identification.

Part 2 – Parts and Material

  - The Hunter Pro-Spray shall be available in the following options

| Pro-Spray | Pro-Spray PRS30 | Pro-Spray PRS40 |
| --- | --- | --- |
| PROS-00 | PROS-00-PRS30 | PROS-00-PRS40 |
| PROS-00-R | PROS-00-PRS30-R | PROS-00-PRS40-R |
| PROS-02 | PROS-04-PRS30 | PROS-04-PRS40-CV |
| PROS-03 | PROS-04-PRS30-CV | PROS-04-PRS40-CV-R |
| PROS-04 | PROS-04-PRS30-CV-R | PROS-04-PRS40-CV-F |
| PROS-04-CV | PROS-04-PRS30-CV-F | PROS-04-PRS40-CV-F-R |
| PROS-04-CV-R | PROS-04-PRS30-CV-F-R | PROS-06-PRS40-CV |
| PROS-06 | PROS-06-PRS30 | PROS-06-PRS40-CV-R |
| PROS-06-CV | PROS-06-PRS30-CV | PROS-06-PRS40-CV-F |
| PROS-06-CV-R | PROS-06-PRS30-CV-R | PROS-06-PRS40-CV-F-R |
| PROS-12 | PROS-06-PRS30-CV-F | PROS-12-PRS40-CV |
| PROS-12-CV | PROS-06-PRS30-CV-F-R | PROS-12-PRS40-CV-R |
| PROS-12-CV-R | PROS-12-PRS30 | PROS-12-PRS40-CV-F |
| PROS-06-SI | PROS-12-PRS30-CV | PROS-12-PRS40-CV-F-R |
| PROS-12-SI | PROS-12-PRS30-CV-R |  |
|  | PROS-12-PRS30-CV-F |  |
|  | PROS-12-PRS30-CV-F-R |  |
|  | PROS-06-SI-PRS30 |  |
|  | PROS-12-SI-PRS30 |  |

- Specification Descriptions

  - PROS-00 = shrub

  - PROS-02 = 2in (5 cm) pop-up

  - PROS-03 = 3in (7.5 cm) pop-up

  - PROS-04 = 4in (10 cm) pop-up

  - PROS-06 = 6in (15 cm) pop-up

  - PROS-12 = 12in (30 cm) pop-up

  - -SI = side inlet model for 6in and 12in (15 cm and 30 cm)

  - -CV = includes factory-installed check valve, “CHECK VALVE” designated on body cap

  - -PRS30 = pressure-regulated to 30 psi (2.1 bar; 210 kPa), brown body cap

  - -PRS40 = pressure-regulated to 40 psi (2.8 bar; 280 kPa), gray body cap

  - -F = includes factory-installed FloGuardTM technology, “FLOGUARD” designated on body cap

  - -R = includes factory-installed reclaimed water purple cap

- Cap Description

- The Pro-Spray cap shall be made of a UV resistant ABS material.

- Standard and pressure-regulated models shall be color-coded for field identification.

  - The standard Pro-Spray shall have a black cap.

  - Pro-Spray PRS30 model shall have a brown cap with “PRS30” marked on the top.

  - Pro-Spray PRS40 model shall have a gray cap with “PRS40” marked on the top.

- When specified for reclaimed water applications, the cap will be molded in purple.

- An optional snap-on cap molded in UV resistant purple acetal shall be available to indicate the use of reclaimed water.

- When specified as factory-installed check valve, the cap shall have “CHECK VALVE” marked on the top.

- When specified with the FloGuard feature, the cap shall have “FLOGUARD” marked on the top.

- Seal Description

- The Pro-Spray shall have a pressure-activated, chemical and chlorine resistant, UV stable wiper seal that cleans debris from the riser while it retracts.

- The seal shall be molded around a rigid plastic ring to prevent seal deformation.

- The seal shall prevent the pop-up from sticking in the up position.

- The seal shall seal the riser stem to the cap under normal operating pressures.

- The seal shall be removable from the cap for easy service and be replaceable.

- Body

  - The body of the Pro-Spray shall be constructed of corrosion and UV-resistant ABS.

  - Standard Pro-Spray and PRS30 models in 6 and 12 in (15 and 30 cm) shall be available with a ½-inch FNPT side inlet option.

  - The inlet size shall be ½” thread.

- Riser Description

- The riser of the sprinkler shall be constructed of abrasion and UV-resistant ABS.

- The riser shall be adjustable for pattern alignment using a two-piece ratchet mechanism.

- The riser shall be compatible with female-threaded nozzles.

- A stainless steel spring shall be assembled with the riser for positive retraction when the irrigation cycle is complete.

- Flush Plug Description

- The Pro-Spray shall have a factory-installed, removable flush plug with a pull-up tab that shall prevent debris from entering the sprinkler during installation.

- Material: polyethylene.

- Check Valve Description

- The Pro-Spray shall have the option of including either a factory-installed or field-installed drain check valve.

- The standard Pro-Spray models with check valve shall check up to 10 ft (3 m) in elevation change.

- The pressure-regulated Pro-Spray models with check valve shall check up to 14 ft (4.3 m) in elevation change.

- When specified with the check valve, the cap shall have “CHECK VALVE” marked on the top.

- FloGuardTM Technology Description

- Pressure-regulated Pro-Spray models with check valve shall include the option of a factory-installed FloGuardTM feature installed in the riser to reduce flow in the event of a missing nozzle.

  - Pro-Spray Dimensions

- Shrub models

  - PROS-00

  - Height = 1.5 in (4 cm)

  - Inlet size = ½ in

- PROS00PRS30 and PROS00PRS40

  - Height = 4.5 in (11 cm)

  - Inlet size = ½ in

- PROS-02

  - Retracted height = 4 in (10 cm)

  - Pop-up height = 2 in (5 cm)

  - Exposed diameter = 2.25 in (5.7 cm)

  - Inlet size = ½ in

- PROS-03

  - Retracted height = 5 in (12.5 cm)

  - Pop-up height = 3 in (7.5 cm)

  - Exposed diameter = 2.25 in (5.7 cm)

  - Inlet size = ½ in

- PROS-04 models

  - Retracted height = 5.875 in (15.5 cm)

  - Pop-up height = 4 in (10 cm)

  - Exposed diameter = 2.25 in (5.7 cm)

  - Inlet size = ½ in

- PROS-06 models

  - Retracted height = 8.75 in (22.5 cm)

  - Pop-up height = 6 in (15 cm)

  - Exposed diameter = 2.25 in (5.7 cm)

  - Inlet size = ½ in

- PROS-12 models

  - Retracted height = 16.125 in (41 cm)

  - Pop-up height = 12 in (30 cm)

  - Exposed diameter = 2.25 in (5.7 cm)

  - Inlet size = ½ in

  - Warranty

    - 5 year warranty period

Part 3 – Function and Operation

- Operating Pressure

- The Pro-Spray shall operate from 15 to 100 psi (1.0 to 7.0 bar; 100 to 700 kPa)

- To activate the pressure-regulator in PRS30 and PRS40 models, a minimum of 10 psi differential is required to achieve the desired output pressure.

  - Pressure Regulation

- When specified as PRS30 or PRS40, the sprinkler shall have a pressure-regulating device as an integral part of the pop-up riser.

- The PRS30 models will prevent fogging or misting of the nozzle spray pattern by maintaining a constant outlet pressure of 30 PSI (2.1 bar; 210 kPa) with inlet pressures of up to 100 PSI (7.0 bar; 700 kPa), regardless of the nozzle installed.

- The PRS40 models will provide optimal distribution uniformity of the MP Rotator by maintaining a constant outlet pressure of 40 PSI (2.8 ba;, 280 kPa) with inlet pressures of up to 100 PSI (7.0 bar; 700 kPa), regardless of the rotator installed.

  - System Flushing

- The flush plug shall allow the system to be flushed before installing the nozzle.

- The flush plug shall open as the stem extends and close when the stem is in the retracted position, prevent debris from entering the riser.

- The flush plug shall have directional flushing action that allows the water to escape in only one direction.

- When specified with the FloGuardTM feature, the flush plug shall include a spacer to keep the FloGuardTM feature deactivated for a complete system flush.

  - Drain Check Valve

- For standard Pro-Spray models, the check valve will hold up to 10 ft (3 m) of elevation.

- For pressure-regulated models, the check valve will hold up to 14 ft (4.3 m) of elevation.

- The check valve can be factory or field-installed in pop-up models.

  - FloGuardTM Technology

- In the event of a missing nozzle, the FloGuard feature shall reduce the flow from the riser to a low-flow 10 ft (3 m) tall stream indicating the need for maintenance.