HUNTER SRV

ELECTRIC REMOTE-CONTROL VALVE

The valve shall be a normally closed, electronically-actuated, hydraulic, remote-control valve. The valve will be capable of operating between___________and_____________PSI (bars; kPa) with a flow range of between________and________GPM (m3/hr; l/m). Pressure loss shall be ____________PSI (bars; kPa) maximum at_______GPM (m3/hr; l/m).

The valve shall be available in a globe configuration. The valve shall be available with an optional, non-rising stem-type, manual flow control mechanism. This mechanism will be operable with a slotted screwdriver or with a removable flow-control handle. When specified for use with reclaimed water, a reclaimed water identifier handle shall be available.

The solenoid shall be a 24 volt, 60 cycle unit with 370mA inrush current and 190mA holding current. BSP models shall use a 24 volt 50 cycle solenoid with a 475 mA inrush current and 230 mA holding current. When specified, the unit shall be equipped with a DC latching solenoid for use with 12-volt battery-operated controllers. The solenoid shall be an encapsulated, one-piece unit with captive plunger.  It shall be equipped with manual internal bleed capability to release the upper chamber water to the downstream piping, allowing the valve to open.

The body shall be molded of non-corrodible PVC. The bonnet shall be of non-corrodible A.B.S. The diaphragm shall be equipped with a serrated disc to screen water-borne contaminants, preventing them from reaching the metering area, and bleed ports of the valve. A perforated diaphragm support ring shall fit into the valve body just below the diaphragm to relieve stress on the diaphragm when the valve is closed.

The valve shall have a 1-inch Female National Pipe Thread (FNPT) inlet and outlet. When specified, the valve shall have a 1-inch slip inlet and outlet or 1-inch Female British Standard Pipe thread inlet and outlet. All valve parts shall be serviceable after installation by unscrewing the bonnet screws, and removing the bonnet from the valve body to access the internal components. This may be accomplished without removing the valve body from the line.

The valve shall carry a two-year, exchange warranty (not prorated).

The valve shall be manufactured for Hunter Industries Incorporated, San Marcos, California.