- I-20 PRB Written Specifications

Part 1 – General

1.1  The I-20 PRB is a pressure-regulated rotary gear-driven sprinkler for residential and commercial applications. It features a non-strippable drive, automatic arc return, non-reversing 360° capabilities, FloStop® control, a drain check valve, water-efficient nozzles that can be easily changed, and simple arc adjustment.

Part 2 – Parts and Material

2.1  The I-20 PRB shall be available in the following versions: Shrub, 4" (10 cm) plastic riser, 4" (10 cm) stainless steel riser, 6" (15 cm) plastic riser, 6" (15 cm) stainless steel riser.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with 30 nozzle choices in flow rates from 0.36 to 9.8 GPM (0.07 to 2.22 m3/hr; 1.2 to 36.0 l/min).

- The sprinkler shall be equipped with a flow shutoff device.

- The sprinkler shall be equipped with a non-strippable gear drive.

- The sprinkler shall be equipped with automatic arc return.

- The arc of the sprinkler shall be adjustable from 50° to 360°.

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

- The sprinkler shall be equipped with a standard drain check valve to hold back up to 10' 
(3 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

The sprinkler shall be equipped with a 45 PSI (3.1 bar; 310 kPa) regulator in the body.

2.2.  Rotor dimensions

A. I-20-00

- Overall height: 8¾" (22 cm)

- Exposed diameter: 1¾" (4.5 cm)

- Inlet size: ¾" NPT

B. I-20-04

- Pop-up height: 4" (10 cm)

- Overall height: 8¼" (22 cm)

- Exposed diameter: 1¾" (4.5 cm)

- Inlet size: ¾" NPT

C. I-20-06

- Pop-up height: 6" (15 cm)

- Overall height: 10⅝" (27 cm)

- Exposed diameter: 1¾" (4.5 cm)

- Inlet size: ¾" NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 55 to 100 PSI (1.7 to 4.5 bar; 170 to 450 kPa)

3.2  Radius: 17' to 46' (4.9 to 14.0 m)

3.3  Flow rate: 0.36 to 9.8 GPM (0.07 to 2.22 m3/hr; 1.2 to 36.0 l/min)

3.4  Precipitation rate: 0.4 in/hr (10 mm/hr) approximately

3.5  Nozzle trajectory: Standard = 25° approximately, Low-angle = 13° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.