- I-25 Written Specifications

Part 1 – General

1.1 The I-25 is a rotary gear-driven sprinkler for commercial applications. It features a non-strippable drive, automatic arc return, non-reversing 360° capabilities, a drain check valve, water-efficient nozzles that can be easily changed, simple arc adjustment, and optional reclaimed water ID.

Part 2 – Parts and Material

2.1  The I-25 shall be available in the following versions: 4" (10 cm) plastic riser, 4" (10 cm) stainless steel riser, 6" (15 cm) plastic riser, 6" (15 cm) stainless steel riser.

- The sprinkler shall be constructed from corrosion- and impact-resistant ABS plastic.

- The sprinkler shall be equipped with a durable, stainless steel riser spring.

- The sprinkler shall be available with 12 nozzle choices in flow rates from 3.8 to 31.5 GPM (0.82 to 7.24 m3/hr; 13.6 to 120.2 l/min).

- The sprinkler shall be equipped with a non-strippable gear drive.

- The sprinkler shall be equipped with automatic arc return.

- The arc of the sprinkler shall be adjustable from 50° to 360°

- The sprinkler shall be equipped with through-the-top arc adjustment.

- The sprinkler shall be equipped with a QuickCheck™ arc mechanism.

- The sprinkler shall be equipped with a factory-installed rubber cover.

- The sprinkler shall be equipped with a standard drain check valve to hold back up to 10' 
(3 m) of elevation.

- The sprinkler shall be available in a reclaimed water ID version.

- The sprinkler shall be available in a high-speed version.

2.2.  Rotor dimensions

A. I-25-04

    - Pop-up height: 4" (10 cm)

    - Overall height: 7⅞" (20 cm)

    - Exposed diameter: 1⅞" (5 cm)

    - Inlet size: 1" (25 mm) NPT

B. I-25-06

    - Pop-up height: 6" (15 cm)

    - Overall height: 10¼" (26 cm)

    - Exposed diameter: 1⅞" (5 cm)

    - Inlet size: 1" (25 mm) NPT

2.3  Warranty

A.  The sprinkler shall carry a five-year exchange warranty.

Part 3 – Function and Operation

3.1  Recommended pressure range: 40 to 100 PSI (2.5 to 7.0 bar; 250 to 700 kPa)

A.  Operating pressure range: 40 to 100 PSI (2.5 to 7.0 bar; 250 to 700 kPa)

3.2  Radius: 37' to 71' (11.9 to 21.6 m)

3.3  Flow rate: 3.8 to 31.5 GPM (0.82 to 7.24 m3/hr; 13.6 to 120.2 l/min)

3.4  Precipitation rate: 0.4 in/hr (15 mm/hr) approximately

3.5  Nozzle trajectory: 25° approximately

© 2019 Hunter Industries Inc. Hunter, the Hunter logo, and all other trademarks are property of Hunter Industries, registered in the U.S. and other countries.