- The Hunter Eco-Wrap shall be pre-assembled and constructed of a sheet of special polypropylene fleece matting with two rows of fleece-wrapped, pressure-compensating, non-draining inline emitter tubing.   The wrapped emitter tubing shall be joined to the top of the fleece mat in regular intervals so as to provide uniform spacing in the lines at 14 in (35 cm) distance and to facilitate uniform installation. The wrapped emitter tubing shall have a nominal outside diameter of 17mm and shall have 0.6 GPH (2.3 l/h) emitters uniformly spaced at 12 in (30 cm) intervals. This tubing shall be made of landscape grade tubing with a nominal wall thickness not less than 1.1 mm. Each roll of Eco Mat 17 shall be 32 in (0,80 m) in width and 295 ft (76 m) in length, providing 785 sq ft (60 sq m) of coverage.  Each roll of Eco-Mat 17 shall be able to hold 0.5 gallons (1,89 liters) of water at maximum saturation. The rolls shall be delivered with solid caps inserted to protect the tubing from debris during installation.

- The Eco-Mat subsurface irrigation system shall be manufactured for Hunter Industries Incorporated, San Marcos, California.