HUNTER I-90

INSTITUTIONAL SERIES
ADJUSTABLE-ARC OR FULL-CIRCLE
GEAR-DRIVEN SPRINKLER

The sprinkler shall be of the gear-driven, rotary type, capable of covering a _________ foot (meter) radius at _________ PSI (bars; kPa) with a discharge rate of _________ GPM (m3/hr; l/min). The sprinkler shall be available with six (6) interchangeable, color-coded nozzles discharging 29.8 GPM to 69.4 GPM (6.77 to 15.76 m3/hr; 113 to 263 l/min).  The sprinkler shall be available in both full-circle and adjustable part-circle configurations. The adjustable part-circle unit shall be minutely adjustable from 40° to 360°. The adjustable unit shall be adjustable in all phases of installation (i.e., before installation, after installation while static, and after installation while in operation). The sprinkler shall be equipped with a manual stator to ensure constant rotation speed

The sprinkler shall be equipped with a drain check valve to prevent low head drainage. The sprinkler shall have a minimum of 3-inch (7.6-cm) pop-up stroke to bring the rotating nozzle turret into a clean environment. The sprinkler shall have a rubber cover firmly attached to the top of the riser. When specified, the sprinkler shall have a cover molded of purple Alcryn rubber to indicate the use of reclaimed water.

The sprinkler shall have an exposed surface diameter after installation of 3-1/2 inches (8.9 cm) and have an overall height of 11 inches (28 cm). The unit shall have a 1-1/2 inch Female National Pipe Thread (FNPT) inlet. When specified, the unit shall have a 1-1/2 inch Female British Standard Pipe Thread inlet.

The sprinkler shall be serviceable after installation by unscrewing the body cap, removing the riser assembly, and extracting the inlet filter screen.

The body of the sprinkler shall be constructed of corrosion resistant, impact resistant, heavy-duty A.B.S. It shall have a stainless steel spring for positive retraction of the riser when irrigation is complete. The sprinkler shall carry a five-year, exchange warranty (not prorated).

The sprinkler shall be manufactured by Hunter Industries Incorporated, San Marcos, California.