- PLD-LOC fittings are made to be compatible with ½” polyethylene tubing and emitterline having an inside diameter range from .520” to .620” (13.3 mm to 15.8 mm).  PLD-LOC fittings are made of UV-resistant, glass reinforced polypropylene material for durability and resistance to cracking/breaking.  The large threaded swivel connectors assist with making installs easy and leak-proof.  Fittings can be disassembled from the tubing and reused.  Fittings are brown-colored to blend in more readily with both the landscape and with brown dripline.  Operating pressure is from 0 to 60 psi (0 to 4.1 bar, 0 to 413 kPa).  Use with Hunter Professional Landscape Dripline (PLD) and Thick Walled Polyethylene (TWPE) products.

- Available Models:

- Coupling: PLD-LOC CPL

- Elbow: PLD-LOC ELB

- Tee: PLD-LOC TEE

- ½” MPT adapter: PLD-LOC 050

- ¾” MPT adapter: PLD-LOC 075

- Flush Cap: PLD-LOC CAP

- Female Hose Swivel: PLD-LOC FHS

- PLD-LOC fittings carry a 2-year warranty.