Provide low-volume point-source emission devices, to efficiently deliver irrigation water in close proximity to the plant. The emitters shall have three options for installation to supply: Self-piercing barb, 10/32 thread and ½" thread. External surfaces shall be constructed from UV-resistant acetyl materials and be self-flushing to minimize clogging.

Emitters shall all be color coded by flow rate for ease of identification:

•	Blue emitter indicates a flow rate of 0.5 GPH (1,89 l/hr)

•	Black emitter indicates a flow rate of 1.0 GPH (3,79 l/hr)

•	Red emitter indicates a flow rate of 2.0 GPH (7,57 l/hr)

•	Tan emitter indicates a flow rate of 4.0 GPH (15,14 l/hr)

•	Orange emitter indicates a flow rate of 6.0 GPH (22,71 l/hr)

Emitters shall also be marked with the flow rates in both Imperial and Metric units on the emitter. Emitters shall be pressure compensating over the pressure range of 20 to 50 PSI (1,4 to 3,5 bar) with consistent flow rate per emitter.

Self-piercing barb inlet emitters with self-piercing barb inlet permit one-step insertion into ½" (13 mm) or ¾" (19 mm) drip tubing when installed with Pocket Punch or HEMT tools.

Self-Piercing Barb Models:

•	HE-050-B (Blue); 0.5 GPH (1.9 l/hr)

•	HE-10-B (Black); 1.0 GPH (3.8 l/hr)

•	HE-20-B (Red); 2.0 GPH (7.6 l/hr)

•	HE-40-B (Tan); 4.0 GPH (15.1 l/hr)

•	HE-60-B (Orange); 6.0 GPH (22.7/hr)

10/32 Male Threaded Models:

•	HE-050-T (Blue); 0.5 GPH (1.9 l/hr)

•	HE-10-T (Black); 1.0 GPH (3.8 l/hr)

•	HE-20-T (Red); 2.0 GPH (7.6 l/hr)

•	HE-40-T (Tan); 4.0 GPH (15.1 l/hr)

•	HE-60-T (Orange); 6.0 GPH (22.7/hr)

½" Female Threaded Models:

•	HEB-05 (Blue); 0.5 GPH (1.9 l/h)

- HEB-10 (Black); 1.0 GPH (3.8l/hr)

•	HEB-20 (Red); 2.0 GPH (7.6 l/hr)

- HEB-40 (Tan); 4.0 GPH (15.1 l/hr)

- HEB-60 (Orange); 6.0 GPH (22.7 l/hr)

Emitter shall be made by Hunter Industries Incorporated.