The inline emitter shall be welded to the inner circumference of the polyethylene tubing. The inline emitter shall have dual outlet ports, 180° apart, ensuring only one port has contact with the ground when the tubing is installed at grade and mulched over.

Hunter’s PROFESSIONAL LANDSCAPE DRIP LINE (PLD) emitter shall pressure compensate by lengthening the emitter’s turbulent flow path. The emitter shall be cylindrical in shape and provide surface area for filtration throughout 360° of its outer circumference.  This increased filtration surface area shall assure that the water that enters the inline emitter can always come from the upper half, or cleanest part of the flow path in the polyethylene tubing regardless of how the inline tubing lays on the ground.

Hunter Reclaimed Professional Landscape Drip tubing shall be purple in color and conform to an Nominal .5575” Inside Diameter, Nominal .045” Wall Thickness and shall be compatible with all industry standard 17MM fittings (both barbed and compression) Hunter Reclaimed Professional Drip tubing shall have factory installed, pressure-compensating, inline emitters installed every [12] or [18] or [24] inches (30, 45, or 60 cm). OR

Hunter Reclaimed Professional Landscape Drip tubing shall have factory installed, pressure-compensating, inline emitters with spacing as indicated on drawings.

The flow rate from each installed inline emitter shall be a consistent [0.6] or [1.0] gallons per hour (2.35 or 3.75 LPH) when inlet pressure is between 15 and 50 psi (1.0 to 3.5 bar).

OPERATIONAL PRESSURE RANGE OF 15-50PSI (1.0 to 3.5 bar)

Models:

Pressure-Compensating, Reclaimed Professional Landscape Drip line for non-potable water systems:

Hunter RECLAIMED PROFESSIONAL LANDSCAPE DRIP LINE (PLD) with emitters and spacing as shown on construction drawings.

- Hunter Reclaimed Professional Drip Tubing PLD -06-12-R (0.6GPH emitters spaced 12 inches on-center) (2.35 LPH emitters spaced 30 cm on-center)

- Hunter Reclaimed Professional Drip Tubing PLD -06-18-R (0.6GPH emitters spaced 18 inches on-center) (2.35 LPH emitters spaced 45 cm on-center)

- Hunter Reclaimed Professional Drip Tubing PLD -06-24-R (0.6GPH emitters spaced 24 inches on-center) (2.35 LPH emitters spaced 60 cm on-center)

- Hunter Reclaimed Professional Drip Tubing PLD -10-12-R (1.0 GPH emitters spaced 12 inches on-center) (3.75 LPH emitters spaced 30 cm on-center)

- Hunter Reclaimed Professional Drip Tubing PLD -10-18-R (1.0 GPH emitters spaced 18 inches on-center) (3.75 LPH emitters spaced 45 cm on-center)

- Hunter Reclaimed Professional Drip Tubing PLD -10-24-R (1.0 GPH emitters spaced 24 inches on-center) (3.75 LPH emitters spaced 60 cm on-center)

PLD shall carry a two-year, exchange warranty (not prorated).

The Landscape Drip Line shall be manufactured by Hunter Industries Incorporated, San Marcos, California.